▼ Product Features:

● Dual channel pulse signal output,90degree phase difference, can detect rotary speed and direction.

● Built-in polarity and short circuit protection.

● Wide temperature range.

- Good EMC anti-interference ability.

- High IPgrade:IP67.

- Mainly used in electric tour car, electric golf carts, electric forklifts and other fields.

▼ Parameters and specifications:

| Character | Parameter Values | Instruction |
| --- | --- | --- |
| Fixing gap | 0~2mm,typical fix gap 1mm | When the rotary frequency is high, the installation clearance shall be as small as possible. |
| Target tooth width | 2 mm±0.1 mm | Tooth module m=1.25, Iron Gear |
| Target groove width | 2 mm±0.1 mm | Tooth module m=1.25, Iron Gear |
| Tooth depth | >3mm | Tooth module m=1.25, Iron Gear |
| Tooth width | >8mm | Tooth module m=1.25, Iron Gear |
| Working temperature | -40~140℃ | Continue operating temperature |
| Service voltage | DC4.5~24V; DC10~30V |  |
| Max reverse voltage | 60VDC |  |
| Operating current | 16mA |  |
| Load current | <80mA |  |
| Signal output form | square wave, collector, open collector | 90°phase difference between two signal A and B channels. when open collector output, the inner pull-up resistor is 5KΩ |
| AB phase-difference | 90±20° | Tooth module m=1.25 |
| Output high Level | >(VCC-0.5V) |  |
| Output low level | <0.5V |  |
| Response frequency | 1~20KHz |  |
| Signal rise time | <10us |  |
| Signal fall edge time | <1us |  |
| IP grade | IP67 | Can be operated in liquid environment |
| Connecting form | 4 wires /AMP | 4 wires: Yellow-VCC, Blue-B, White-A, Black-GND |

▼ Electrical Connection：

When do the no load test, the A pulse and B pulse must series 5K resistance separately with VCC.

▼ Output Signal：

When pulse wheel is rotated forward, A pulse will advance B pulse 90degree.。

When pulse wheel is rotated backward, B pulse will advance A pulse 90degree.

▼ Test Method (unit: mm):

The best target gear tooth:

- The material: Magnetic conductivity metal materials

2、  Size：tooth width:2mm, tooth space width: 2mm, tooth high: 5mm